Instructions For Use

Instructions For Using The Keys

① On/off (power identification)

Long press to turn on the system within one second, and long press the button to shut down in the power-on state; Power on the default automatic shutdown function (10min), short press the button until the power indicator disappears at the top left of the screen, then turn off the automatic shutdown function, and press again to turn on.

② REF

Short press to set the current test result as the reference value, make a relative power measurement, and the screen will display the relative power dB value; Press and hold to the screen to display the word RJ45 to open the RJ45 test, and press and hold again until the word  RJ45 disappears on the screen to close the RJ45 test.

③ LIGHT

Short press to turn on the red light, press again to flash, press again to close, cycle in turn, when turned on, the word 650 is displayed at the bottom of the screen;

Press and hold until the word Cal appears on the left side of the screen to enter the calibration mode, and press and hold again until the word Cal disappears to exit and save the calibration value.

| Wavelength range (nm) | 800~1700 |
| --- | --- |
| Probe type | InGaAs |
| Measuring range (dBm) | -70~+6 or -50~+26 |
| Uncertainty | ±5% |
| Standard wavelength (nm) | 850/980/1270/1300/1310/1490/ 1550/1577/1625 |
| Display resolution | Linear display: 0.001%~0.1% Logarithmic display: 0.01dBm |
| Red light output wavelength | 650nm |
| Red light output power (mW) | 1/5/10/15/20/25/30 (Optional) |
| Real-time frequency test range | 0~3000Hz |
| USB input (or charging) | 5V/1A |
| Operating temperature (℃) | -10~+60 |
| Storage temperature (℃) | -25~+70 |
| Automatic shutdown time | 10min |
| Battery continuous operating time | ≥48h |
| Power supply | 2 dry batteries, 3.7V lithium battery (optional) or external 5V input |
| Meter dimensions (mm) | 69*119*31 |
| Meter net weight (g) | 157 |

| Calibration mode operating instructions |  |  |
| --- | --- | --- |
| Key | Operate | Function |
| LIGHT | Long press | Enter calibration mode or exit and save the calibration value |
|  | Short press | Set the current wavelength calibration value to zero |
| λ | Short press | Switch the calibration wavelength |
| REF | Short press | The current wavelength calibration value increases by 0.05dB |
| UNIT | Short press | The current wavelength calibration value is reduced by 0.05dB |

④ LED

Short press on/off lighting LED; Press and hold until the sun icon appears at the top left of the screen to turn on the backlight, and press and hold again until the sun logo disappears to turn off the backlight.

⑤ UNIT

Short press to switch data display units between dB and mW (nW/uW); Press and hold again to enter the real-time frequency test mode, and press again to exit.

⑥ λ

Switch the test wavelength (with wavelength memory function).

Absolute power, relative power and linear power values are converted as follows:

P Absolute Power = 10LgP Linear Power /1mW

P Relative Power = P Absolute Power - P Reference Power

Technical Indicators